Integrated Hardware & ML Pipeline — Strategic Plan (Simple & Actionable)

This document merges both the hardware pipeline and the ML pipeline into one unified view for building a helmet-embedded navigation and safety system, with a lightweight helmet node and a bike-mounted hub.

Hardware Pipeline — Strategic Plan

Goal: lightweight helmet node + bike-mounted hub (camera, heavier compute & battery) so the helmet stays light and the system is fully independent of a phone.

[Helmet Node]  <--- short breakaway cable / BLE --->  [Bike Hub]
  (light, low-power)                                (heavy compute, camera, modem, battery)

Phases

- Partition & Spec

  - On-helmet: microphone, bone-conduction speaker, IMU, GNSS receiver, small MCU (sensor fusion & TTS playback), quick-release connector.

  - On-hub (bike): camera(s), routing engine, heavy ML inference, LTE voice modem, main battery, storage, OTA updates.

  - Deliverable: one-page partition spec.

- Helmet Node Prototype

  - Components: Raspberry Pi Zero 2 W or MCU, GNSS, IMU, MEMS mic, bone-conduction speaker, quick-release connector.

  - Output: helmet node that streams GNSS/IMU and plays TTS.

  - Failure modes: overheating, noisy audio, antenna issues. Mitigation: low-power firmware, bone-conduction testing, external antenna placement.

- Bike Hub Prototype

  - Components: Jetson Nano / Raspberry Pi 4, 1–2 cameras, LTE USB modem, 20–30Wh battery, SSD for maps.

  - Output: hub that computes routes, hosts modem call flow, streams instructions to helmet.

  - Failure modes: modem drop, camera misalignment. Mitigation: fallback retries, calibration.

  - Hub: add Google API client

    - Components: API key stored securely on hub, HTTPS client library, small cache (Redis or file cache), quota monitor.

    - Actions: call Google Routes/Directions for route + traffic ETA, Roads API for snap-to-road map-matching when GNSS is noisy.

    - Deliverable: hub/google_client module that returns route alternatives, ETA, and map-matched traces.

    - Failure mode & mitigation: API quota or network failure → use cached routes or local GraphHopper/Valhalla; implement exponential backoff + alert in logs.

- Integration & Safety

  - Actions: external antenna routing, breakaway cable (mechanical + fused), waterproof connectors.

  - Failure mode: cable snag → neck hazard. Mitigation: magnetic/snap connectors.

  - Cache Google responses with expiry (e.g., 5–15 minutes for traffic-dependent ETA; route geometry maybe longer but respect Google TOS). Implement automated expiry and cache eviction.

  - Add local fallback path: if !google_available: compute_local_reroute().

- Field Testing & Iteration

  - Test: audio clarity, GNSS accuracy, dead-reckoning drift, emergency call triggers.

  - Output: field report + tuning list.

- MVP Packaging

  - Finalize enclosure, cable routing, charging, safe shutdown.

  - Output: prototype ready for extended testing.

MVP Components

- Helmet node: MCU/SBC, u-blox GNSS, IMU (ICM-20948), bone-conduction transducer, MEMS mic, TTS board, quick-release connector.

- Bike hub: Jetson Nano / Pi4, camera(s), LTE modem, 20–30Wh battery, SSD/microSD.

ML Pipeline — Strategic Plan

Goal: assist routing & rerouting, provide ETA, detect anomalies, with light inference on helmet and heavy inference on hub.

[Data Capture] -> [Preprocess & Label] -> [Train & Validate] -> [Package & Deploy] -> [Monitor & Iterate]

Phases

- Data Design & Collection

  - Inputs: camera frames, GNSS, IMU, speed, route ID, events (deviation, delay, reroute).

  - Helmet: high-frequency IMU & GNSS. Hub: video + route logs.

  - Output: synchronized trip logs.

- Preprocessing & Labeling

  - Actions: map-match GNSS, align camera, label events.

  - Tools: Python/pandas, py-map-match, ffmpeg.

  - Failure mode: timestamp misalign. Mitigation: NTP sync.

- Modeling (Train / Validate)

  - Dead-reckoning (Kalman filter on helmet).

  - ETA correction: LightGBM (hub/cloud).

  - Reroute decision: start rule-based → small classifier (XGBoost).

  - Vision: object detection (Jetson optional).

  - Metrics: ETA MAE, reroute precision/recall.

- When to call Google (hub decision rules)

- Debounce reroute calls: call Google at most once every 5–10 seconds and only when deviation > threshold (e.g., 20–30 m) or predicted delay > X.

- Apply rule: auto-apply recommended if eta_saving > 3 minutes & confidence >= 0.8. Otherwise prompt driver.

  - How to use Google outputs for ML

- Use Google ETAs and snapped traces as labels or features when training your ETA-correction/ reroute policy models in the cloud/hub (but respect caching/TOS).

- If you store Google-derived data for training, enforce short expiry windows and anonymize location data per Google terms and privacy policy.

  - Packaging

- Treat Google calls as an augmenting signal; keep core safety-critical logic (dead-reckoning, voice prompts, emergency call) local on helmet/hub so system remains functional offline.

- Deployment

  - On-hub: full models, map-matching, routing engine.

  - On-helmet MCU: Kalman filter, ETA adjuster, TTS trigger logic.

  - Format: TFLite/ONNX, OTA updates.

- On-Device Inference & Decision Flow

  - Helmet: runs Kalman, ETA fallback, TTS.

  - Hub: routing, reroute, auto-call.

  - Output: local safety-critical functions remain operational even if hub disconnects.

- Monitoring & Iteration

  - Log predictions vs outcomes.

  - Retrain periodically with ride data.

  - Output: retrained OTA models.

Minimal ML Recipe for MVP

- Rule-based routing + Kalman on helmet.

- Collect ride data for 2–4 weeks.

- Train LightGBM ETA model.

- Deploy lightweight model to hub.

Data & Control Flow (Simple)

[Helmet sensors: GNSS, IMU, Mic, Speaker] --(telemetry + TTS control over breakaway/BLE)-> [Bike Hub: Camera, Router, Modem, Battery]
        ^                                                                               |
        |----------------- TTS instructions / alerts (voice) ---------------------------|

- Helmet sends telemetry (10Hz IMU, 1Hz GNSS).

- Hub computes route + ML inference, sends TTS prompts.

- Hub makes LTE call if thresholds are triggered.

Recommended 6-Step Strategic Roadmap

- Spec & Partition (1–3 days).

- Helmet Node Prototype (1–2 weeks).

- Hub Prototype (2–3 weeks).

- Integration (2–3 weeks).

- Data Collection & Basic ML (4+ weeks).

- Refine & Safety Testing (ongoing).

Success Criteria (Key Metrics)

- ETA MAE ≤ target minutes.

- Reroute precision: most reroutes improve ETA.

- Helmet audio intelligibility >90% in wind.

- Dead-reckoning drift <50m over 2 minutes.

Short MVP Must-Haves

- Turn-by-turn voice (helmet TTS).

- Dead-reckoning fallback (helmet Kalman).

- Offline routing (hub with precomputed lists).

- Auto-call on delay (hub LTE modem).

- Helmet remains lightweight (battery & compute offloaded).